Goal

Add two bits:

a

b

A normal (classical) 1-bit addition gives:

| a | b | SUM | CARRY |
| --- | --- | --- | --- |
| 0 | 0 | 0 | 0 |
| 0 | 1 | 1 | 0 |
| 1 | 0 | 1 | 0 |
| 1 | 1 | 0 | 1 |

2. Qubits

Use 3 qubits:

q0 = a

q1 = b

q2 = carry (initially 0)

Initial state:

|000>

3. Load inputs (a and b)

Qubits always start at 0.
To put a 1 into a qubit, we flip it using the X gate.[0 1

1 0]

if a = 1 → flip q0

if b = 1 → flip q1

Example: a=1, b=1:

|000> → |100> → |110>

Now the qubits hold the inputs.

4. Compute the CARRY using CCNOT

CCNOT rule:

Flip the third qubit only when the first and second are both 1.

So:

q0=1 AND q1=1 → flip q2

Example starting from |110>:|110> → |111>

This correctly produces:

CARRY = 1 (since 1 AND 1 = 1)

If a or b is 0, CCNOT does nothing.

5. Compute SUM using CNOT gates

CNOT rule:

Flip the target qubit only if the control qubit is 1.

We want:

SUM = a XOR b

For XOR, one qubit must flip depending on the other.

Two CNOTs do this:

CNOT(q0 → q1)

CNOT(q1 → q0)

These two flips create the XOR behavior.

Example (a=1, b=1):

Start after CCNOT:

|111>

First CNOT (q0→q1)

q0 = 1 → flip q1:

Second CNOT (q1→q0)

q1 = 0 → no flip:

|101> (final)

Meaning:

SUM = q0 = 0

CARRY = q2 = 1

Correct for:

1 + 1 = 10 (binary)

6. Final Output

From the final qubits:

Sum = q0

Carry = q2

Case 1: a = 0, b = 0

Start:

∣0,0,0⟩

CCNOT:

c=0⊕(0⋅0)=0

State:

∣0,0,0⟩

CNOT:

s=0⊕0=0

Final:

∣0,0,0⟩

Sum = 0, Carry = 0

Case 2: a = 0, b = 1

Start:

∣0,1,0⟩

CCNOT:

c=0⊕(0⋅1)=0

CNOT:

s=0⊕1=1

Final:

∣0,1,0⟩

Sum = 1, Carry = 0

Case 3: a = 1, b = 0

Start:

∣1,0,0⟩

CCNOT:

c=0⊕(1⋅0)=0

CNOT:

s=1⊕0=1

Final:

∣1,1,0⟩

Sum = 1, Carry = 0

Case 4: a = 1, b = 1

Start:

∣1,1,0⟩

CCNOT:

c=0⊕(1⋅1)=1

State becomes:

∣1,1,1⟩

CNOT:

s=1⊕1=0

Final:

∣1,0,1⟩

Sum = 0, Carry = 1